##### Revision History

| Revision | Release date | comment |
| --- | --- | --- |
| 1.0 | 2011/03/17 | First issue. |
| 2.0 | 2011/05/26 | Modify for RTL8192/88CE MP mode. |
| 3.0 | 2012/05/30 | Modify for RTL8189ES/EUS MP mode. |

##### Introduction

The RTL8189ES Linux driver supports driver based Mass Production functions. Customers can utilize these functions to do EMI test and other simple TX and RX test. Under RTL8189ES turnkey, we use Linux utility “iwpriv” to get and set I/O control to WLAN driver. The following commands are executed under Linux command prompt. The MP functions will only operate after the WLAN interface is opened.

##### Usage

- Start MP mode:

“iwpriv wlan0 mp_start”

After executing this command, WLAN driver enters MP mode and stops transmitting and receiving any packets. All connection with other stations will be broken. Beacon transmitting is also stopped. If the original state is client mode, the roaming will be stopped.

- Stop MP mode:

“iwpriv wlan0 mp_stop”

After executing this command, WLAN driver will stops transmitting and receiving packets initialized by other commands. It will not return to normal operation mode. System should close the WLAN interface and open again to get WLAN work normally.

- Set Tx rate:

“iwpriv wlan0 mp_rate rate”

- rate		sets the rate of Tx packets (i.e., 2 for 1M, 4 for 2M, 11 for 5.5M, …, 108 for 54M, 128 for MCS0, 129 for MCS1, …, 142 for MCS15)

Set the data rate of continuous transmitting.

- Set operational channel:

“iwpriv wlan0 mp_channel channel”

- channel 	sets the channel to send and receive packets

Set the operational channel of transmitting and receiving packets.

- Set operational bandwidth:

“iwpriv wlan0 mp_bandwidth [40M=40m,shortGI=sgi]”

- 40m 	sets the operational bandwidth, 1 for 40M mode, 0 for 20M mode

- sgi 		sets guard interval of transmitting MCS packet, 1 for short GI, 0 for long GI

Set the operational bandwidth for transmitting and receiving packets. Set the guard interval for transmitting MCS packets. If the parameters are not given, the default action is set bandwidth to 20M mode and long GI to transmitting packets.

- Set Tx power:

“iwpriv wlan0 mp_txpower [patha=x,pathb=y]”

- x 		sets Tx power level for path A

- y 		sets Tx power level for path B

Set the transmitting power level of path A and path B. If the parameters are not given, the driver will set Tx power according to the flash setting.

- Set antenna for Tx:

“iwpriv wlan0 mp_ant_tx ant”

- ant 		sets the operational antenna for Tx, a for antenna A, b for antenna B, ab for antenna A and B

Set the operational antenna for Tx on the target board.

- Set antenna for Rx:

“iwpriv wlan0 mp_ant_rx ant”

- ant 		sets the operational antenna for Rx, a for antenna A, b for antenna B, ab for antenna A and B

Set the operational antenna for Rx on the target board.

- Start air Rx mode:

“iwpriv wlan0 mp_arx start/stop”

- start 	clears counters and start to accumulate Rx packets

- stop 	stops counts and show the statistics

This command is for air receiving test. Use start command to clear all the counters and start to accumulate the received packets. Use stop command to stop counting and show the statistics of correct packets and CRC-error packets.

- Start continuous Tx mode:

“iwpriv wlan0 mp_ctx count=n,background,stop,pkt,cs,stone”

- t 		sets the number of seconds to send packets

- n 		sets the number of packets to send

- background sends packets in background mode

- stop 	stops the background sending

- pkt 		sends packet tx, i.e., not sent by hardware

- cs		sends carrier suppression

- stone 	sends single tone

This command is for continuous transmitting test. Use time command to assign the time to send packets. Use count command to assign the number of packets to send. If both of time and count are not specified, the sending function will continue infinitely. It can be stopped when any key is pressed (should be specifically implemented in other platform) while background command is not specified. If cs is specified, the sending signal will be a carrier suppression signal. Use stone command to send single tone signal for frequency testing. If stone is specified, the sending signal will be single tone and not a distinguishable packet any more. Use background command to tell driver to send packets in background. The command line control will return and packet sending is continuous. It can be stopped by stop command. By default, packets will be sent by hardware for shorter duty cycle. If pkt is specified, packets will be sent by software.

- Query air Rx statistics:

“iwpriv wlan0 mp_query”

This command is for packet counting. Under packet transmitting, use this command to get the number of packets being transmitted. Under receiving, use this command to get the statistics of correct packets and CRC-error packets.

- Reset air Tx/Rx statistics:

“iwpriv wlan0 mp_reset_stats”

This command is for reset the packet counts. Under packet transmitting, use this command to reset the number of packets after the transmitted or receiving, use this command to reset the statistics of correct packets and CRC-error packets.

##### Example Command:

- Continuous Tx testing:

(Execute the following commands after WLAN interface is normally opened)

ifconfig wlan0 up							Enable Device for MP operation

iwpriv wlan0 mp_start						enter MP mode

iwpriv wlan0 mp_channel 1					set channel to 1 . 2, 3, 4~11 etc.

iwpriv wlan0 mp_bandwidth 40M=0,shortGI=0  set 20M mode and long GI

iwpriv wlan0 mp_ant_tx a					select antenna A for operation

iwpriv wlan0 mp_txpower patha=44,pathb=44   set path A and path B Tx power level

iwpriv wlan0 mp_rate 108					 set OFDM data rate to 54Mbps,ex: CCK 1M = 2, CCK 5.5M = 11, KK, OFDM54M = 108 N Mode: MCS0 = 128, MCS1 = 129…..etc.

iwpriv wlan0 mp_ctx background				start continuous Tx

iwpriv wlan0 mp_ctx stop					stop continuous Tx

iwpriv wlan0 mp_stop						exit MP mode

ifconfig wlan0 down						close WLAN interface

- Continuous Packet Tx testing:

(Execute the following commands after WLAN interface is normally opened)

ifconfig wlan0 up							Enable Device for MP operation

iwpriv wlan0 mp_start						enter MP mode

iwpriv wlan0 mp_channel 1					set channel to 1 . 2, 3, 4~11 etc.

iwpriv wlan0 mp_bandwidth 40M=0,shortGI=0  set 20M mode and long GI

iwpriv wlan0 mp_ant_tx a					select antenna A for operation

iwpriv wlan0 mp_txpower patha=44,pathb=44   set path A and path B Tx power level

iwpriv wlan0 mp_rate 108					 set OFDM data rate to 54Mbps,ex: CCK 1M = 2, CCK 5.5M = 11, KK, OFDM54M = 108 N Mode: MCS0 = 128, MCS1 = 129…..etc.

iwpriv wlan0 mp_ctx background,pkt		start packet continuous Tx

iwpriv wlan0 mp_ctx stop					stop continuous Tx

iwpriv wlan0 mp_stop						exit MP mode

ifconfig wlan0 down						close WLAN interface

- Count Packet Tx testing:

(Execute the following commands after WLAN interface is normally opened)

ifconfig wlan0 up							Enable Device for MP operation

iwpriv wlan0 mp_start						enter MP mode

iwpriv wlan0 mp_channel 1					set channel to 1 . 2, 3, 4~11 etc.

iwpriv wlan0 mp_bandwidth 40M=0,shortGI=0  set 20M mode and long GI

iwpriv wlan0 mp_ant_tx a					select antenna A for operation

iwpriv wlan0 mp_txpower patha=44,pathb=44   set path A and path B Tx power level

iwpriv wlan0 mp_rate 108					 set OFDM data rate to 54Mbps,ex: CCK 1M = 2, CCK 5.5M = 11, KK, OFDM54M = 108 N Mode: MCS0 = 128, MCS1 = 129…..etc.

iwpriv wlan0 mp_ctx count=%d,pkt	    	Number of packets start packet Tx

iwpriv wlan0 mp_ctx stop					stop continuous Tx

iwpriv wlan0 mp_stop						exit MP mode

ifconfig wlan0 down						close WLAN interface

- Carrier suppression testing:

(Execute the following commands after WLAN interface is normally opened)

ifconfig wlan0 up							Enable Device for MP operation

iwpriv wlan0 mp_start						enter MP mode

iwpriv wlan0 mp_channel 1					set channel to 1 . 2, 3, 4~11 etc.

iwpriv wlan0 mp_bandwidth 40M=0,shortGI=0  set 20M mode and long GI

iwpriv wlan0 mp_ant_tx a					select antenna A for operation

iwpriv wlan0 mp_txpower patha=44,pathb=44   set path A and path B Tx power level

iwpriv wlan0 mp_rate 108					 set OFDM data rate to 54Mbps,ex: CCK 1M = 2, CCK 5.5M = 11, KK, OFDM54M = 108 N Mode: MCS0 = 128, MCS1 = 129…..etc.

iwpriv wlan0 mp_ctx cs,background		start sending carrier suppression signal

iwpriv wlan0 mp_ctx stop					stop continuous Tx

iwpriv wlan0 mp_stop						exit MP mode

ifconfig wlan0 down						close WLAN interface

- Single Tone Tx testing:

(Execute the following commands after WLAN interface is normally opened)

ifconfig wlan0 up							Enable Device for MP operation

iwpriv wlan0 mp_start						enter MP mode

iwpriv wlan0 mp_channel 1					set channel to 1 . 2, 3, 4~11 etc.

iwpriv wlan0 mp_bandwidth 40M=0,shortGI=0  set 20M mode and long GI

iwpriv wlan0 mp_ant_tx a					select antenna A for operation

iwpriv wlan0 mp_txpower patha=44,pathb=44   set path A and path B Tx power level

iwpriv wlan0 mp_rate 108					 set OFDM data rate to 54Mbps,ex: CCK 1M = 2, CCK 5.5M = 11, KK, OFDM54M = 108 N Mode: MCS0 = 128, MCS1 = 129…..etc.

iwpriv wlan0 mp_ctx background,stone      start sending single tone signal

iwpriv wlan0 mp_ctx stop					stop sending single tone signal

iwpriv wlan0 mp_stop						exit MP mode

ifconfig wlan0 down						close WLAN interface

- Air Rx testing:

(Execute the following commands after WLAN interface is normally opened)

ifconfig wlan0 up						Enable Device for MP operation

iwpriv wlan0 mp_start					enter MP mode

iwpriv wlan0 mp_bandwidth 40M=1,shortGI=0

set 40M mode and long GI

iwpriv wlan0 mp_channel 6				set channel to 6

iwpriv wlan0 mp_ant_rx ab				select all 2 antennas for operation

iwpriv wlan0 mp_arx start				start air Rx

iwpriv wlan0 mp_query					get the statistics

iwpriv wlan0 mp_arx stop				stop air Rx and show the statistics

iwpriv wlan0 mp_stop					exit MP mode

ifconfig wlan0 down					close WLAN interface